# Hawk 100R

The Hawk 100R set includes: Engine with temp probe, RPM sensor, Mounting lugs, Glow plug, ECU, RX cable, Cable set for glow plug, Fuel pump with cable set, HDT, Transport casing, Manual fuel valve, O-ring spare part for starter, Fuel & Propane valve, Hose for Kerosene, Hose for propane, Engine users manual, Warranty paper, Three years warranty.

# Technical spec.

Hawk 100R technical figures

Example of typical data:

Thrust range                                       0.6 - 11.0 kg  / 1.3 - 24.4  lbs	+/- 10%

Pressure ratio                                     1.2 - 3.8

Rpm                                               	60 000 - 175 000 rpm

EGT                                                   390 - 760° (max 800) Celsius

Fuel consumption                                30 - 260g / minute

Fuel                                         	  	Kerosene, Jet A1 or Diesel (EN590) with 1.5% oil

Oil quality                                       	Two stroke outboard oil

Engine controll                                    Fadec full auto start

Starting gas                                    	Propane butane mixture

Battery                                          	LiPo 7.4,  2000mAh (Not incl.)

Mounting                                            Lugs with fuel / propane connectors

Compressor                                    	Radial, transonic in-house design

Turbine                                          	Radial

Combustor                                         Reverse flow type

Engine weight                                 	1650 gram (engine and starter)

Airborne Install weight                         Aprox. 2000 gram

All data in ISA conditions.

Unique features for the Hawk 100R

- Unique rotor damping system

- Milled in house designed compressor

- Mounting lugs of solid aluminium with fuel connectors

- Lowest fuel consumption on the market (2,1 liters will last for 10 minutes flying in normal sport / scale aircraft)

- Fast spool times

- Three years warranty

# Fuel figures

Already when passing 15N we have the best SFC on the market.

| # Performance The Hawk 100R has, what we believe, the fastest spool time on the market. Factory settings for the Hawk 100R is around 4 seconds from idle to max power. However you have the possibility to change the setting for times down to around 2,5 seconds. This gives you the spool time from half throttle (aprox 50N of thrust) to maximum throttle in around half a second. This is a result due to very high combined turbine and compressor efficiency.. |  |
| --- | --- |

# Dimensions

# Hawk 240R

The Hawk 240R set includes:

Engine with temp probe, RPM sensor, Mounting lugs, Glow plug,

ECU, RX cable, Cable set for glow plug, Fuel pump with cable set,

HDT,  Manual fuel valve, O-ring spare part for starter,

Fuel & Propane valve, Hose for Kerosene, Hose for propane,

On a USB memory:Engine users manual, Warranty paper,

Three years warranty.

# Technical spec.

Hawk 240R technical figures

Example of typical data:

Thrust range                                       0.8 - 24.0 kg  / 1.8 - 52,9  lbs	+/- 10%

Pressure ratio                                	  1.2 - 4,6

Rpm                                                   39 000 - 132 000 rpm

EGT                                              	 400 - 760° ( max 800) Celsius

Fuel consumption                                 65 - 515g / minute

Fuel                                         	  	Kerosene, Jet A1 or Diesel (EN590) with 1.5% oil

Oil quality                                       	Two stroke outboard oil

Engine controll                                    Fadec full auto start

Starting gas                                    	Propane butane mixture

Battery                                          	LiPo 7.4,  2000mAh (Not incl.)

Mounting                                            Lugs with fuel / propane connectors

Compressor                                    	Radial, transonic in-house design

Turbine                                          	Radial

Combustor                                         Reverse flow type

Engine weight                                     2800 gram (engine and starter)

Airborne Install weight                         Aprox. 3100 gram

All data in ISA conditions.

Unique features for the Hawk 240R

- Unique rotor damping system

- Milled in house designed compressor

- Mounting lugs of solid aluminium with fuel connectors

- Lowest fuel consumption on the market (4,5 liters will last for 10 minutes flying in normal sport / scale aircraft)

- Fast spool times

- Three years warranty

| # Fuel Figures. |  |
| --- | --- |

| # Dimensions. |  |
| --- | --- |

# Hawk 190R

The Hawk 190R set includes:

Engine with temp probe, RPM sensor, Mounting lugs, Glow plug,

ECU, RX cable, Cable set for glow plug, Fuel pump with cable set,

HDT, LiPo battery, Manual fuel valve, O-ring spare part for starter,

Fuel & Propane valve, Hose for Kerosene, Hose for propane,

Engine users manual, Warranty paper,

Three years warranty.

# technic spec.

Hawk 190R technical figures

Example of typical data:

Thrust range                                       0.5 - 19.0 kg  / 1.3 - 41.9  lbs	+/- 10%

Pressure ratio                                	  1.2 - 3.9

Rpm                                                   39 000 - 130 000 rpm

EGT                                                   390 - 760° ( max 800) Celsius

Fuel consumption                                 50 - 390g / minute

Fuel                                         	  	Kerosene, Jet A1 or Diesel (EN590) with 1.5% oil

Oil quality                                       	Two stroke outboard oil

Engine controll                                    Fadec full auto start

Starting gas                                    	Propane butane mixture

Battery                                          	LiPo 7.4,  2000mAh (Not incl.)

Mounting                                            Lugs with fuel / propane connectors

Compressor                                    	Radial, transonic in-house design

Turbine                                          	Radial

Combustor                                         Reverse flow type

Engine weight                                     2800 gram (engine and starter)

Airborne Install weight                         Aprox. 3100 gram

All data in ISA conditions.

Unique features for the Hawk 190R

- Unique rotor damping system

- Milled in house designed compressor

- Mounting lugs of solid aluminium with fuel connectors

- Lowest fuel consumption on the market (3,2 liters will last for 10 minutes flying in normal sport / scale aircraft)

- Fast spool times

- Three years warranty

| # fuel figures_. |  |
| --- | --- |

| # Dimensions_ Hawk 190R. |  |
| --- | --- |

# Hawk 290R

The Hawk 290R set includes:

Engine with temp probe, RPM sensor, Mounting lugs, Glow plug,

ECU, RX cable, Cable set for glow plug, Fuel pump with cable set,

HDT,  Manual fuel valve, O-ring spare part for starter,

Fuel & Propane valve, Hose for Kerosene, Hose for propane,

On a USB memory:Engine users manual, Warranty paper,

Three years warranty.

| # Technical spec 290R Hawk 290R technical figures Example of typical data: Thrust range 0.8 - 29.0 kg / 1.8 - 52.9 lbs +/- 10% Pressure ratio 1.2 - 4,7 Rpm 39 000 - 132 000 rpm EGT 400 - 780° ( max 800) Celsius Fuel consumption 84 - 659g / minute Fuel Kerosene, Jet A1 or Diesel (EN590) with 1.5% oil Oil quality Two stroke outboard oil Engine controll Fadec full auto start Starting gas Propane butane mixture Battery LiPo 7.4, 2000mAh (Not incl.) Mounting Lugs with fuel / propane connectors Compressor Radial, transonic in-house design Turbine Radial Combustor Reverse flow type Engine weight 2800 gram (engine and starter) Airborne Install weight Aprox. 3100 gram All data in ISA conditions.. |  |
| --- | --- |

# Dimensions 290R